|  |  | Unit ID: 2144 |
| --- | --- | --- |
| Domain |  | METAL FABRICATION-CORE |
| Title: |  | Weld cast iron using shielded metal arc welding process. |
| Level: 3 |  | Credits: 8 |

# Purpose

This unit standard is intended for those who weld cast iron using the shielded metal arc welding process. People credited with this unit standard can prepare materials, equipment; and weld cast iron; conduct post weld inspection and housekeeping.

This unit standard is intended for those who work as Welders.

Special Notes

1. Entry information

Prerequisite:

- 228 - Apply safety rules and regulations in a metal fabrication work   environment

2. 	To demonstrate competence, at a minimum, evidence is required of performing one butt. 	These tasks should be performed ensuring correct identification of requirements and 	finishing of the tasks, correct 	selection and use of appropriate processes, tools and 	equipment and completing all work to specification.

3. 	Assessment evidence may be collected from a real workplace in which metal fabrication 	operations are carried out.

4. 	Performance of all elements in this unit standard must comply with manufacturers’ 	instructions and workplace specific requirements.

5.	‘Specifications refer to any or all of the following: manufacturers’ specifications and 	recommendations, site and workplace specific requirements.

6. 	Regulations and legislation relevant to this unit standard include the following:

- Occupational Health and Safety Regulations No.18, 1997.

- Labour Act 11 of 2007 and all subsequent amendments.

- National Occupational Safety Association Regulations

  - Occupational Health and Safety Regulations No.18, 1997 and all subsequent amendments.

# Quality Assurance Requirements

This unit standard and others within this Subfield may be awarded by institutions which meet the accreditation requirements set by the Namibia Qualifications Authority and the Namibia Training Authority, and which comply with the national assessment and moderation requirements. Details of specific accreditation requirements and the national assessment arrangements are available from the Namibia Qualifications Authority and the Namibia Training Authority on www.nta.com.na.

# Elements and Performance Criteria

Element 1: Plan and prepare for work

Range

Pre work planning includes but is not limited to correct selection of tools and equipment for the task, materials for the task, personal protective equipment for the task and planning the logical sequence of the task. Planning and preparation include but is not limited to worksite inspection, equipment defect identification, assessment of conditions and hazards and determination of work requirements.

# Performance Criteria

  - Work instructions, including plans, specifications, quality requirements and operational details are obtained, confirmed and applied.

  - Safety requirements are followed in accordance with safety plans and policies.

  - Sign and barricade requirements are identified and adhered to.

  - Plant, tools and equipment selected to carry out tasks are consistent with the requirements of the job, checked for serviceability and any faults are rectified or reported prior to commencement.

  - Environmental protection requirements are identified and applied in accordance with environmental plans and regulatory obligations.

Element 2: Prepare materials and equipment

## Range

Test procedures may include but are not limited to; voltage drop, amperage setting, earthing, terminals connection and wire conductivity.

Preparation of materials may include but not limited, pre-heating, fixtures and clamps.

## Performance criteria

2.1	Personal protective equipment is selected and used in line with workplace procedures

2.2 	Welding requirements are identified from work instructions.

2.3       Material is selected, prepared and aligned in line with task requirements.

2.4      Welding equipment is assembled and set as per manufacturers’ recommendation and task requirement.

2.5      Welding machine settings and consumables are identified in line with task requirement and welding procedures specification

Element 3: Repair cast iron components

## Performance criteria

3.1      Shielded metal arc welding risks and hazards are identified, eliminated and minimized.

3.3       Welding consumables are selected in line with welding procedures specification.

3.4  Handling methods of welding consumable are explained as per manufacturer recommendation.

3.5	Welding techniques are applied in line welding procedure specification.

3.6	Equipment shut down procedures are followed in line with manufacturer’s recommendation

3.7       Distortion prevention methods are identified and applied.

3.8	Work pieces are welded in line with job requirements and welding procedure specification

3.9	Equipment shut down procedures are followed in line with manufacturer’s recommendation

Element 4: Perform post weld inspection and housekeeping

Range

Inspection of work piece may include but not limited to metal control, lack of penetration, undercut, pinholes, inclusion and porosity.

Performance criteria

  - Welds are cleaned and dressed in line welding procedure specification and construction drawing.

  - Plant, tools and equipment are cleaned, checked, maintained and stored in accordance with manufacturers' recommendations and standard work practices.

  - Work area is cleared and materials disposed of, reused or recycled in accordance with legislation, regulations and job specifications.

# Registration Data

| Subfield: | Manufacturing Engineering |
| --- | --- |
| Date first registered: |  |
| Date this version registered: |  |
| Anticipated review: | 2024 |
| Body responsible for review: | Namibia Training Authority |