|  | Unit ID: | 241 |
| --- | --- | --- |
| Domain | METAL FABRICATION |  |
| Title: | Brazing and silver soldering metals using oxy-acetylene equipment as part of metal fabrication |  |
| Level: 3 |  | Credits: 4 |

Purpose

This unit standard is intended for those who join metals using brazing and silver soldering using oxy-acetylene equipment as part of metal fabrication. People credited with these unit standard can prepare materials, equipment and join them using oxy-fuel brazing, silver soldering processes; conduct post inspection and housekeeping.

This unit standard is intended for those who work as Welders.

# Special Notes

- Entry information

Prerequisite:

- Unit 228 - Apply safety rules and regulations in a metal fabrication work   environment

- To demonstrate competence, at a minimum, evidence is required of joining work pieces to specifications, on any of the materials listed in Special Note 6, using oxy-acetylene equipment as part of metal fabrication. These tasks should be performed ensuring correct identification of requirements and finishing of the tasks, correct selection and use of appropriate processes, tools and equipment and completing all work to specification.

- Assessment evidence may be collected from a real workplace or a simulated real workplace in which there is a high degree of realism that replicates commercial or domestic workplace setting in which welding operations are carried out.

- Performance of all elements in this unit standard must comply to specifications requirement and manufacturers’ /workplace recommendations.

- Specifications refer to any or all the followings: manufacturer’s recommendations, site and work specification requirements.

- Metals to be joined may include but not limited to mild steel, cast iron and non-ferrous metals.

- Regulations and legislation relevant to this unit standard include the following:

  - Occupational Health and Safety Regulations No.18, 1997.

  - Labour Act 11 of 2007 and all subsequent amendments.

  - National Occupational Safety Association Regulations

Quality Assurance Requirements

This unit standard and others within this Subfield may be awarded by institutions which meet the accreditation requirements set by the Namibia Qualifications Authority and the Namibia Training Authority, and which comply with the national assessment and moderation requirements. Details of specific accreditation requirements and the national assessment arrangements are available from the Namibia Qualifications Authority on www.namqa.com and the Namibia Training Authority on www.nta.com.na.

# Elements and Performance Criteria

Element 1: Plan and prepare for work

Range

Pre work planning includes but is not limited to correct selection of tools and equipment for the task, materials for the task, personal protective equipment for the task and planning the logical sequence of the task. Planning and preparation include but is not limited to worksite inspection, equipment defect identification, assessment of conditions and hazards and determination of work requirements.

# Performance Criteria

  - Work instructions, including plans, specifications, quality requirements and operational details are obtained, confirmed and applied.

  - Safety requirements are followed in accordance with safety plans and policies.

  - Sign and barricade requirements are identified and adhered to.

  - Plant, tools and equipment selected to carry out tasks are consistent with the requirements of the job, checked for faults and rectified or reported prior to commencement.

  - Environmental protection requirements are identified and applied in accordance with environmental plans and regulatory obligations.

Element 2: Select, prepare materials and equipment

Performance Criteria

2.1	Oxy-acetylene equipment for brazing and soldering are identified, selected and tested prior to operation.

2.2	Personal protective equipment is selected and inspected.

2.3	The required filler material and fluxes are selected in line with job requirements.

2.4	Work pieces are prepared in line with job requirements.

2.5 	Equipment is assembled in line with manufacturer recommendation.

Element 3: Join materials

Range

Brazing and Soldering processes may include but not limited to heating with oxy-acetylene flame, soldering bolt and iron.

Performance Criteria

  - Personal protective equipment is used in line with workplace procedures.

  - Safety precautions are adhered during brazing and soldering process.

  - Flame is set in accordance with base metal characteristics.

  - Pre-heating and post-heat treatment process is performed in line with welding procedure specification.

  - Work pieces are fit and tacked in position as per drawing or work instruction.

  - Work pieces are secured to minimise distortion during brazing and soldering.

  - Work pieces are brazed or soldered in positions as per specifications and requirements.

  - Brazing and soldering are performed in line with welding techniques.

Element 4: Conduct post brazing and silver soldering inspection

Range

Non-destructive test may include but not limited to inspection of metal control, lack of penetration and porosity.

Destructive tests may include but not limited to bend test and peel test.

Evidence of non-destructive test is required for assessment purposes.

Performance Criteria

4.1	Work pieces are inspected for welding defects.

4.2 	Joined base metals are inspected for imperfection in line with work instruction.

Element 5: Perform housekeeping

# Performance Criteria

  - Plant, tools and equipment are cleaned, checked, maintained and stored in accordance with manufacturers' recommendations and standard work practices.

  - Work area is cleared and materials disposed of, reused or recycled in accordance with legislation, regulations and job specifications.

Registration Data

| Subfield: | Manufacturing Engineering |
| --- | --- |
| Date first registered: |  |
| Date this version registered: |  |
| Anticipated review: | 2024 |
| Body responsible for review: | Namibia Training Authority |